ARP SPOOFING &

MAN-IN-THE-MIDDLE ATTACKS

Defense Strategies & Senior-Level Countermeasures

Classification: Confidential — Internal Security Use

Project: ARP-Spoofing-MITM

Author: Kitsana Thuekoh

Date: May 2026

# 1. Executive Summary

ARP (Address Resolution Protocol) Spoofing is one of the most persistent and effective Layer 2 network attacks in the cybersecurity threat landscape. By poisoning the ARP cache of target devices, a malicious actor can silently insert themselves into all communications between a victim and a gateway — a position known as a Man-in-the-Middle (MITM) — enabling traffic interception, credential theft, session hijacking, and data manipulation without the victim's knowledge.

This document provides a senior-level analysis of ARP Spoofing and MITM attack mechanics, followed by a comprehensive, defense-in-depth strategy encompassing network architecture controls, host-level hardening, active detection mechanisms, enterprise tooling, and governance considerations. It is intended for security architects, network engineers, SOC leads, and senior penetration testers responsible for securing enterprise environments.

Key Risk: A single undefended switch port is sufficient for a threat actor to establish a full MITM position on a LAN segment, intercepting all unencrypted traffic including credentials, session tokens, and sensitive business data.

# 2. Threat Overview

## 2.1 How ARP Works

ARP is a stateless, unauthenticated protocol operating at Layer 2 of the OSI model. When a device needs to communicate with another host on the same subnet, it broadcasts an ARP Request asking "Who has IP x.x.x.x? Tell IP y.y.y.y". The owner of that IP replies with its MAC address. This mapping is cached locally in the ARP table for future use.

The critical vulnerability: ARP has no authentication mechanism. Any device on the network can send an unsolicited ARP Reply (a "gratuitous ARP") and hosts will accept it, updating their ARP cache accordingly.

## 2.2 ARP Spoofing Attack Chain

- Attacker gains access to the LAN segment (physical, wireless, or via compromised endpoint)

- Attacker sends forged ARP Replies to the victim: "The gateway MAC address is [attacker MAC]"

- Attacker sends forged ARP Replies to the gateway: "The victim MAC address is [attacker MAC]"

- Both devices update their ARP caches with the attacker's MAC address

- All traffic between victim and gateway now flows through the attacker

- Attacker enables IP forwarding to maintain victim's connectivity (avoiding detection)

- Attacker captures, reads, modifies, or injects traffic at will

## 2.3 Attack Variants

### Classic ARP Poisoning

Bilateral cache poisoning as described above. Tools: arpspoof, ettercap, Bettercap.

### ARP Flooding / Cache Overflow

Flooding a switch with random ARP entries to overflow the CAM table, causing it to fail-open and broadcast all traffic (similar effect to a MITM). Tools: macof (dsniff suite).

### Gratuitous ARP Abuse

Sending unsolicited ARP Replies that update caches without a prior ARP Request, enabling silent cache poisoning with no network anomaly that triggers standard IDS rules.

### ARP MITM with SSL Stripping

Once in the MITM position, the attacker downgrades HTTPS connections to HTTP using tools like sslstrip, intercepting credentials from sites that lack HSTS enforcement.

## 2.4 Attack Impact Matrix

| Impact Category | Attack Vector | Severity | Business Risk |
| --- | --- | --- | --- |
| Credential Theft | HTTP/FTP/Telnet sniffing | Critical | Account takeover, data breach |
| Session Hijacking | Cookie/token interception | Critical | Unauthorised access to systems |
| Data Manipulation | Payload injection in transit | High | Data integrity compromise |
| Reconnaissance | Full traffic capture | High | Network mapping, intel gathering |
| DoS via Poisoning | Blackhole routing | Medium | Service disruption |

# 3. Defence-in-Depth Strategy

Effective defence against ARP Spoofing requires a layered approach. No single control is sufficient. The following strategy covers six layers: network infrastructure hardening, switch-level controls, encryption, detection and monitoring, host hardening, and governance.

## 3.1 Layer 2 Network Infrastructure Controls

### Dynamic ARP Inspection (DAI)

DAI is the primary enterprise-grade control for ARP Spoofing prevention. Available on managed enterprise switches (Cisco, Juniper, Aruba, etc.), DAI validates ARP packets against a trusted DHCP Snooping binding table before forwarding them.

- Enable DHCP Snooping on all VLANs to build the IP-to-MAC-to-port binding table

- Enable DAI on all untrusted ports (all access ports facing end devices)

- Mark uplink ports and trunk ports as DAI-trusted

- ARP packets that do not match the binding table are silently dropped

- Configure rate limiting on ARP traffic to prevent ARP flooding attacks

Cisco IOS example configuration:

ip dhcp snooping
ip dhcp snooping vlan 10,20,30
ip arp inspection vlan 10,20,30
interface GigabitEthernet0/1
 ip arp inspection limit rate 100

### Port Security

Configure port security on access switches to limit the number of MAC addresses per port and detect MAC flooding attacks:

- Restrict each access port to 1-3 MAC addresses maximum

- Set violation mode to restrict or shutdown on violation

- Enable sticky MAC learning for legitimate devices

- Integrate with 802.1X for authenticated port access

### 802.1X Network Access Control (NAC)

802.1X enforces authentication before any device is permitted to communicate on the network. This prevents unauthorised devices — including attacker machines — from ever obtaining a network connection on a LAN segment.

- Deploy RADIUS server (Cisco ISE, FreeRADIUS, Microsoft NPS) for authentication backend

- Enforce certificate-based EAP-TLS for strongest authentication

- Use MAB (MAC Authentication Bypass) only for legacy devices, with tight controls

- Place unauthenticated devices into a quarantine VLAN with no access to production resources

### VLAN Segmentation

Aggressive VLAN segmentation limits the blast radius of any ARP Spoofing attack to a single network segment:

- Separate servers, workstations, IoT, and guest traffic into distinct VLANs

- Never allow users on the same VLAN as servers without explicit business justification

- Disable VLAN trunking on all access ports (prevent VLAN hopping)

- Use private VLANs (PVLANs) in high-security zones to prevent host-to-host communication within the same VLAN

## 3.2 Encryption Controls

### Enforce TLS/HTTPS Everywhere

Encrypting all traffic in transit ensures that even if an attacker successfully establishes a MITM position, intercepted data is unreadable:

- Enforce HTTPS with TLS 1.2 minimum (TLS 1.3 preferred) on all web applications

- Implement HTTP Strict Transport Security (HSTS) with preloading to prevent SSL stripping

- Deploy HSTS on all internal web applications, not just public-facing ones

- Use certificate pinning in mobile and thick client applications

### VPN for Inter-Segment and Remote Traffic

Route all sensitive inter-site and remote access traffic through encrypted VPN tunnels:

- Deploy IPSec or WireGuard VPNs for site-to-site traffic

- Require VPN for all remote access (zero-trust network access model preferred)

- Enforce mutual authentication on VPN connections

### SSH and Encrypted Management Protocols

- Disable Telnet, FTP, and SNMP v1/v2c on all network devices — replace with SSH v2, SFTP, and SNMPv3

- Use SSH host key verification and alert on host key changes

- Implement jump servers / bastion hosts for all network device management

## 3.3 Detection & Monitoring Controls

### Network-Based ARP Monitoring

Deploy dedicated ARP monitoring tools to detect poisoning attempts in real time:

- ArpWatch: monitors ARP traffic and alerts on IP-to-MAC binding changes — deploy on dedicated monitoring interfaces spanning all VLANs

- XArp: Windows/Linux GUI-based ARP monitoring with active probing capabilities

- Zeek (formerly Bro): network security monitor that can detect ARP anomalies via custom scripts

### SIEM Integration & Detection Rules

Integrate ARP telemetry into your SIEM (Splunk, Microsoft Sentinel, Elastic SIEM) with the following detection logic:

- Alert on duplicate IP entries with different MAC addresses in ARP tables

- Alert on gratuitous ARP packets from non-infrastructure devices

- Alert on rapid MAC address changes for the same IP (more than 3 changes in 60 seconds)

- Alert on ARP traffic volume anomalies per port (potential ARP flooding)

- Correlate ARP alerts with authentication failures and lateral movement indicators

SIEM Rule Example: IF ARP_REPLY.src_ip == gateway_ip AND ARP_REPLY.src_mac != known_gateway_mac THEN ALERT CRITICAL — Potential ARP Spoofing targeting gateway

### Network Traffic Analysis (NTA)

Deploy network traffic analysis tools for full packet inspection and behavioural anomaly detection:

- Darktrace / Vectra AI: machine learning-based NTA that detects MITM positioning through traffic flow anomalies

- Zeek + Elastic Stack: open-source NTA pipeline with ARP anomaly detection scripts

- Cisco Stealthwatch (now Secure Network Analytics): flow-based anomaly detection across the enterprise

### Endpoint Detection & Response (EDR)

- Deploy EDR agents (CrowdStrike Falcon, Microsoft Defender for Endpoint, SentinelOne) on all endpoints

- EDR agents can detect local ARP cache manipulation attempts and alert on suspicious arpspoof/ettercap process execution

- Monitor for tools associated with ARP attacks: arpspoof, ettercap, bettercap, scapy

## 3.4 Host-Level Hardening

### Static ARP Entries for Critical Assets

For high-value, low-change assets (servers, domain controllers, firewalls, gateways), configure static ARP entries that cannot be overwritten by dynamic ARP Replies:

- Windows: arp -s <IP> <MAC> or configure via Group Policy / PowerShell

- Linux: ip neigh add <IP> lladdr <MAC> dev eth0 nud permanent

- Network devices: configure static ARP entries on router/gateway interfaces for critical servers

Note: Static ARP does not scale well for dynamic environments. Use in conjunction with DAI rather than as a standalone control.

### Host-Based Firewall & IDS

- Enable host-based firewalls on all endpoints with default-deny outbound rules

- Deploy Snort or Suricata on sensitive servers with ARP anomaly detection rules

- Rule example: alert arp any any -> any any (msg:"ARP Spoofing Detected"; arpop:2; content:"|00 00|"; offset:6; depth:2; sid:1000001;)

### Operating System ARP Hardening

- Linux: set net.ipv4.conf.all.arp_ignore=1 and net.ipv4.conf.all.arp_announce=2 in /etc/sysctl.conf

- Linux: enable arp_filter to prevent incorrect ARP responses from multi-homed hosts

- Windows: use the Windows Defender Credential Guard and modern authentication to reduce credential exposure even if traffic is intercepted

## 3.5 Zero Trust Network Architecture (ZTNA)

The most strategically robust long-term defence against ARP Spoofing and MITM attacks is the adoption of Zero Trust Network Architecture, which assumes no implicit trust based on network location:

- Never trust, always verify: authenticate and authorise every connection request regardless of source network

- Microsegmentation: replace flat VLANs with software-defined perimeters where each workload communicates only with explicitly permitted resources

- Identity-based access: bind access decisions to verified user and device identity, not IP or MAC address

- Mutual TLS (mTLS): enforce certificate-based mutual authentication for all service-to-service communication, rendering MITM positioning ineffective

- Continuous validation: re-authenticate sessions periodically and on anomaly detection, not just at initial connection

ZTNA solutions: Zscaler Private Access, Cloudflare Access, Palo Alto Prisma Access, Microsoft Entra Private Access.

# 4. Incident Response Playbook

When ARP Spoofing is detected or suspected, execute the following response procedure:

## 4.1 Detection & Triage

- Confirm the alert via SIEM / ARP monitoring tool — verify IP-to-MAC binding change is not legitimate (e.g., scheduled NIC replacement)

- Identify the affected VLAN, switch port, and IP/MAC addresses involved

- Cross-reference the rogue MAC address against the asset inventory and 802.1X authentication logs

- Determine scope: is this an isolated incident or indicative of wider lateral movement?

## 4.2 Containment

- Immediately isolate the affected switch port using port shutdown or VLAN reassignment

- Clear poisoned ARP caches on affected hosts: arp -d * (Windows) / ip neigh flush all (Linux)

- Clear ARP caches on gateway/router interfaces for affected segments

- If attacker device is identified: disable its 802.1X port access and quarantine to isolated VLAN

- Block the attacker MAC address at the switch level

## 4.3 Eradication & Recovery

- Conduct forensic capture of the attacker's traffic if legally permissible

- Reset all credentials that may have transited the compromised path (prioritise admin accounts, service accounts, VPN credentials)

- Invalidate all active sessions and force re-authentication across affected systems

- Patch the attack vector: enable DAI, enforce 802.1X on the previously unprotected segment

- Review and update static ARP entries for critical assets

## 4.4 Post-Incident Review

- Conduct a root cause analysis: how did the attacker gain LAN access?

- Review detection gap: why was the attack not detected sooner?

- Update SIEM detection rules and ARP monitoring thresholds based on findings

- Report to relevant stakeholders including CISO, Legal, and (where applicable) regulatory bodies

# 5. Senior-Level Tooling Reference

The following table provides a consolidated reference of tools relevant to ARP Spoofing defence, detection, and red team validation:

| Tool | Category | Platform | Purpose |
| --- | --- | --- | --- |
| ArpWatch | Detection | Linux | Real-time ARP table change monitoring and alerting |
| XArp | Detection | Win/Linux | GUI ARP monitoring with active probing |
| Zeek + Scripts | Detection/NTA | Linux | Network security monitoring with ARP anomaly scripts |
| Cisco ISE | NAC/802.1X | Enterprise | Network access control and DAI enforcement backend |
| Darktrace | AI/NTA | Enterprise | ML-based MITM detection via traffic flow anomalies |
| Splunk / Sentinel | SIEM | Enterprise | ARP alert correlation and SOC dashboard |
| Wireshark | Forensics | Win/Linux | Packet capture for ARP attack confirmation and analysis |
| Bettercap | Red Team | Linux | Authorised ARP spoofing simulation for control validation |
| Snort / Suricata | Host IDS | Linux | Host-based ARP anomaly detection rules |
| CrowdStrike Falcon | EDR | Win/Linux/Mac | Endpoint detection of ARP spoofing tool execution |

# 6. Governance & Compliance Considerations

## 6.1 Regulatory Alignment

ARP Spoofing defences directly support compliance with the following frameworks and regulations:

| Framework / Regulation | Relevant Controls |
| --- | --- |
| PCI DSS v4.0 | Req 1 (Network Security Controls), Req 6 (System Security), Req 10 (Logging & Monitoring) |
| ISO/IEC 27001:2022 | A.8.20 (Network Security), A.8.22 (Segregation of Networks), A.8.16 (Monitoring Activities) |
| NIST CSF 2.0 | PR.AC (Access Control), DE.CM (Continuous Monitoring), RS.AN (Incident Analysis) |
| POPIA (South Africa) | Condition 7 (Security Safeguards) — protecting personal information in transit from interception |
| CIS Controls v8 | Control 12 (Network Infrastructure Management), Control 13 (Network Monitoring & Defence) |

## 6.2 Security Testing Requirements

ARP Spoofing defences must be validated through regular authorised testing:

- Quarterly penetration tests must include Layer 2 attack simulation including ARP Spoofing and VLAN hopping attempts

- Red team exercises should validate MITM detection capabilities and SOC response time

- DAI and 802.1X configurations must be reviewed during annual network security audits

- All ARP monitoring alerting must be tested via purple team exercises to confirm SIEM detection rules fire correctly

## 6.3 Policy Requirements

- Network Security Policy must explicitly prohibit ARP cache modification tools on production networks without change control approval

- All managed switches must have DAI and DHCP Snooping enabled as a baseline configuration standard

- Guest networks must be on isolated VLANs with no Layer 2 adjacency to corporate networks

- Any deviation from 802.1X enforcement must require documented risk acceptance from the CISO

# 7. Summary & Recommendations

ARP Spoofing remains a high-impact, low-complexity attack that is trivially executable by any attacker with LAN access and widely available tools. However, it is also well-understood and highly defensible when appropriate controls are implemented at the infrastructure layer.

The following priority actions are recommended for immediate implementation in any enterprise environment:

| # | Priority | Recommendation | Expected Outcome |
| --- | --- | --- | --- |
| 1 | Critical | Enable DHCP Snooping and DAI on all managed switches | Eliminates majority of ARP Spoofing vectors at the network layer |
| 2 | Critical | Deploy 802.1X NAC on all access ports | Prevents unauthorised devices from joining the network |
| 3 | High | Enforce HSTS and TLS 1.3 across all internal and external services | Renders intercepted traffic unreadable; defeats SSL stripping |
| 4 | High | Deploy ArpWatch with SIEM integration and 24/7 alerting | Detects active ARP Spoofing attempts in real time |
| 5 | High | Implement aggressive VLAN segmentation with private VLANs in critical zones | Limits blast radius of any successful attack |
| 6 | Medium | Configure static ARP entries on gateways for all critical server IPs | Protects highest-value assets from cache poisoning |
| 7 | Medium | Begin Zero Trust Architecture migration roadmap | Eliminates implicit Layer 2 trust as a strategic long-term control |
| 8 | Medium | Conduct quarterly Layer 2 penetration tests to validate controls | Continuous assurance that controls remain effective |

END OF DOCUMENT

vetementsvmnts / ARP-Spoofing-MITM Project — May 2026